8255 Programming Model

| Address | Action |
| --- | --- |
| 0C000h | Port A |
| 0C001h | Port B |
| 0C002h | Port C |
| 0C003h | Control Reg |

Port A - 0C000h

|  |  |  |  |  |  |  |  | Motor | Motor |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FUNCTION | I/O | 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 | DEC |
| Motor Left | Out | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 00 |
| Motor Forward | Out | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 01 |
| Motor Back | Out | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 02 |
| Motor Right | Out | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 03 |

Port B - 0C001h

|  |  | /Echo | +5 | Bat Mon | +5 | Right | Left | Back | Front |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FUNCTION | I/O | 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 | DEC |
| No Hit | In | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |  |
| Front Bumper | In | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 0 |  |
| Back Bumper | In | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 1 |  |
| Left Bumper | In | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |  |
| Right Bumper | In | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 1 |  |

Port C - 0C002h

|  |  | RC F/B | RC LT | RC RT |  |  |  |  | Sonar Init |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FUNCTION | I/O | 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 | DEC |
| Sonar Init | Out | X | X | X | X | 0 | 0 | 0 | 1 |  |
| RC - Right | In | 0 | 0 | 1 | 0 | X | X | X | X |  |
| RC - Left | In | 0 | 1 | 0 | 0 | X | X | X | X |  |
| RC - For/Back | In | 1 | 0 | 0 | 0 | X | X | X | X |  |
|  | In | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  |

Control - 0C003h

| FUNCTION | Mode | 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 | DEC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Port A - Out Port B - In Port C - In (up) Port C - Out (lo) | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 0 |  |
| Port A - Out Port B - In Port C - Out | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |  |
|  | 1 |  |  |  |  |  |  |  |  |  |

Memory Map

| From | To | Memory |
| --- | --- | --- |
| 0 (0000H) | 8K (1FFFH) | RAM |
| 8K (2000H) | 16K (2FFFH) | RAM |
| 16K (4000H) | 32K (7FFFH) | Free |
| 32K (8000H) | 48K (AFFFH) | EPROM |
| 48K (C000H) | (C003H) | 8255 |
|  | Special Locations |  |
| 24 (18H) | Collision detector and direction Front=14, Back=13, Left=11, Right=7 |  |
| 25 (19H) | Sonar no echo if = 100 |  |
| 32 (20H) | Sonar high byte |  |
| 33 (21H) | Sonar low byte |  |
| 138 (8AH) | Timer 0 low byte |  |
| 140 (8CH) | Timer 0 high byte |  |
| 00 - 127 (7F) | Are assume to be INTERNAL RAM |  |
| 128 (80) - 255 (FF) | Are assume to be SFR registers |  |